To map the detected color centroids from the undistorted camera image into the robot base coordinate frame, a planar homography transformation matrix H∈R3x3 is computed. Using the twelve ground-truth marker correspondences listed in Table 1, the projective mapping between image pixel coordinates (u,v) and horizontal workspace coordinates (Xw,Yw) is formulated as:

sXwYw1=Huv1=h11h12h13h21h22h23h31h32h33uv1 # SEQ Equation 2

Where s denotes an arbitrary projective scale factor, and hij represents the elements of the homography matrix estimated via Singular Value Decomposition (SVD) with Random Sample Consensus (RANSAC) outlier filtering. By dehomogenizing the projective coordinates, the explicit Cartesian coordinates (Xw,Yw) dispatched to the motion planner are derived as:

Xw=h11u+h12v+h13h31u+h32v+h33, Yw=h21u+h22v+h23h31u+h32v+h33# SEQ Equation 3

Here, (u, v) represent the horizontal and vertical pixel coordinates of the detected target centroid on the camera image plane, whereas (Xw, Yw) denote the resulting physical Cartesian coordinates in millimeters relative to the robot base coordinate frame. The coefficients h11 through h33 account for perspective rectification, orientation alignment, and spatial scaling between the optical sensor and the workspace table. These computed metric coordinates (Xw, Yw) are directly transmitted to the MoveIt 2 motion planning pipeline to execute deterministic pick-and-place trajectories.